SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN) |
| Weightage: | 30% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | G.THARAKNATHREDDY | Reg.No.: | 192472032 |
| Department: | CSE-AI | Date: |  |

ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

- Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

- Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

- Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

- Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

- Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Code of Conduct:

I G.THARAKNATHREDDY(192472032) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: G.THARAKNATHREDDY

MARKS ALLOCATION

|  | Problem Understanding (20%) | Algorithm Design (30%) | Use of Control Structures (20%) | Pseudocode Structure (10%) | Completeness (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

ANSWERS:

Q1. DMA-Based Data Transfer Algorithm

Problem Understanding

Design a DMA-based algorithm that transfers data directly from an I/O device to main memory. The CPU should configure the DMA controller and handle completion/error interrupts, but should not copy the actual data.

Inputs

- I/O device

- Destination memory address

- Transfer size

- DMA controller

- Interrupt mechanism

Output

- Data successfully transferred to memory

- Transfer completion/error notification

Pseudocode

ALGORITHM DMA_Data_Transfer

INPUT:

Device_ID

Memory_Address

Transfer_Size

OUTPUT:

Transfer_Status

BEGIN

INITIALIZE DMA_Controller

INITIALIZE Interrupt_Controller

// Step 1: Validate request

IF Transfer_Size <= 0 THEN

RETURN "Invalid transfer size"

END IF

IF Memory_Address is not valid THEN

RETURN "Invalid memory address"

END IF

// Step 2: Prepare destination buffer

Allocate Memory_Buffer of Transfer_Size

// Step 3: Configure DMA

DMA.Source       ← Device_ID

DMA.Destination  ← Memory_Buffer

DMA.Length       ← Transfer_Size

DMA.Direction    ← DEVICE_TO_MEMORY

DMA.Mode         ← INTERRUPT_ON_COMPLETION

// Step 4: Enable DMA interrupt

Enable_Interrupt(DMA_INTERRUPT)

// Step 5: Start DMA transfer

DMA.Start()

// CPU does NOT copy the data

CPU_State ← AVAILABLE

// Step 6: Wait for interrupt

WHILE DMA.Transfer_Complete = FALSE DO

IF DMA.Error = TRUE THEN

Disable_Interrupt(DMA_INTERRUPT)

RETURN "DMA transfer failed"

END IF

Perform_Other_CPU_Tasks()

END WHILE

END

DMA Interrupt Service Routine

ISR DMA_Interrupt_Handler

BEGIN

Save_Minimum_CPU_Context()

Status ← DMA.Read_Status()

IF Status = TRANSFER_COMPLETE THEN

Clear_DMA_Interrupt()

Mark_Buffer_Ready()

Notify_Application()

ELSE IF Status = TRANSFER_ERROR THEN

Clear_DMA_Interrupt()

Record_Error()

Notify_Error_Handler()

END IF

Restore_CPU_Context()

RETURN_FROM_INTERRUPT()

END ISR

Optimized Operation

I/O Device

↓

DMA Controller

↓

Main Memory

↓

DMA Completion Interrupt

↓

CPU

The CPU only performs configuration and completion processing, while DMA handles the bulk transfer.

Why this achieves Level 4

- Uses DMA correctly.

- CPU involvement is minimized.

- Includes interrupt handling.

- Handles errors.

- Uses conditions and loops.

- Separates normal processing from the ISR.

- Avoids CPU-based byte-by-byte copying.

Q2. Priority-Based Vectored Interrupt Handling

Problem Understanding

Multiple devices generate interrupts simultaneously. The system must determine which interrupt has the highest priority, identify the correct service routine using a vector table, execute it, and then return to the interrupted task.

Inputs

- Interrupt requests from multiple devices.

- Priority levels.

- Interrupt vector table.

Output

- Correct ISR executed for each interrupt.

Pseudocode

ALGORITHM Vectored_Interrupt_Controller

INPUT:

Interrupt_Request_List

OUTPUT:

Completed_ISR

BEGIN

Initialize_Interrupt_Controller()

FOR EACH Device IN Device_List DO

Assign_Vector(Device)

Assign_Priority(Device)

END FOR

WHILE System_Running = TRUE DO

Pending_List ← Get_Pending_Interrupts()

IF Pending_List is NOT EMPTY THEN

// Select highest-priority interrupt

Highest_Priority ← Find_Highest_Priority(Pending_List)

Vector_Number ← Get_Vector_Number(Highest_Priority)

Dispatch_ISR(Vector_Number)

ELSE

Execute_Normal_Task()

END IF

END WHILE

END

Interrupt Dispatcher

FUNCTION Dispatch_ISR(Vector_Number)

BEGIN

Save_CPU_Context()

ISR_Address ← Interrupt_Vector_Table[Vector_Number]

IF ISR_Address is NULL THEN

Record_Invalid_Interrupt(Vector_Number)

Restore_CPU_Context()

RETURN

END IF

Execute ISR_Address

Restore_CPU_Context()

RETURN

END FUNCTION

Example Priority Table

| Device | Vector | Priority |
| --- | --- | --- |
| Emergency Sensor | 1 | 1 |
| Network Controller | 2 | 2 |
| DMA Controller | 3 | 3 |
| USB Controller | 4 | 4 |
| Keyboard | 5 | 5 |

Here, priority 1 is the highest priority.

If several devices interrupt simultaneously:

Sensor + USB + DMA

↓

Priority Controller

↓

Sensor selected

↓

Vector 1

↓

Sensor_ISR()

Nested Interrupt Extension

ISR Current_Device

BEGIN

Save_Context()

Enable_Higher_Priority_Interrupts()

Handle_Device()

Disable_Higher_Priority_Interrupts()

Restore_Context()

RETURN

END

A higher-priority interrupt can therefore interrupt a lower-priority ISR.

Why this achieves Level 4

The algorithm includes:

- Interrupt prioritization.

- Vector-table lookup.

- ISR dispatch.

- Invalid interrupt handling.

- Nested interrupt capability.

- Loops and conditional statements.

- Normal task execution when no interrupt is pending.

Q3. Dynamic Selection Between Memory-Mapped I/O and I/O-Mapped I/O

Problem Understanding

Different devices may have different latency and bandwidth requirements. The system should select an appropriate I/O mechanism based on workload characteristics.

Inputs

- Required latency.

- Required bandwidth.

- Processor architecture support.

- Device type.

- Access frequency.

Output

- Selected I/O mechanism.

Pseudocode

ALGORITHM Select_IO_Mechanism

INPUT:

Required_Latency

Required_Bandwidth

Access_Frequency

Device_Type

CPU_Supports_IO_Mapped

CPU_Supports_Memory_Mapped

OUTPUT:

Selected_IO_Method

BEGIN

// Check whether memory-mapped I/O is supported

IF CPU_Supports_Memory_Mapped = FALSE THEN

IF CPU_Supports_IO_Mapped = TRUE THEN

RETURN "I/O-MAPPED I/O"

ELSE

RETURN "No supported I/O mechanism"

END IF

END IF

// High-performance devices

IF Required_Bandwidth >= HIGH_BANDWIDTH THEN

Selected_IO_Method ← "MEMORY-MAPPED I/O"

// Very latency-sensitive access

ELSE IF Required_Latency <= LOW_LATENCY THEN

Selected_IO_Method ← "MEMORY-MAPPED I/O"

// Frequently accessed device registers

ELSE IF Access_Frequency >= HIGH_ACCESS_FREQUENCY THEN

Selected_IO_Method ← "MEMORY-MAPPED I/O"

// Legacy/separate I/O-space architecture

ELSE IF CPU_Supports_IO_Mapped = TRUE AND

Device_Type = "LEGACY_IO_DEVICE" THEN

Selected_IO_Method ← "I/O-MAPPED I/O"

ELSE

Selected_IO_Method ← "MEMORY-MAPPED I/O"

END IF

RETURN Selected_IO_Method

END

Decision Logic

Device Requirements

↓

┌───────────┴───────────┐

↓                       ↓

High bandwidth?          Low latency?

↓                       ↓

YES                     YES

↓                       ↓

Memory-Mapped I/O       Memory-Mapped I/O

For a device that uses a dedicated I/O address space on a processor that supports it:

Legacy Device

↓

Separate I/O Address Space

↓

I/O-Mapped I/O

Important Design Note

The selection should not be based on an assumption that one mechanism is universally faster. Actual latency depends on the processor architecture, bus/interconnect, compiler instructions, device implementation, and system configuration.

Therefore, a production implementation should measure actual performance.

Benchmark Extension

FUNCTION Measure_IO_Performance(Method)

BEGIN

Start_Timer()

FOR i ← 1 TO TEST_ITERATIONS DO

Perform_IO_Access(Method)

END FOR

Stop_Timer()

Latency ← Calculate_Average_Latency()

Bandwidth ← Calculate_Bandwidth()

RETURN Latency, Bandwidth

END FUNCTION

The system can compare measured results before selecting the configuration.

Q4. Bus Arbitration for PCIe, USB, and Thunderbolt

Problem Understanding

Multiple devices may request access to shared system resources. The arbitration mechanism should prevent one device from monopolizing available resources while ensuring that high-bandwidth devices receive sufficient service.

The system contains:

- PCIe devices.

- USB devices.

- Thunderbolt devices.

The algorithm must provide:

- Fairness.

- Priority handling.

- Starvation prevention.

- Queue management.

- Resource allocation.

Pseudocode

ALGORITHM Bus_Arbitration

INPUT:

PCIe_Queue

USB_Queue

Thunderbolt_Queue

OUTPUT:

Device_Service

BEGIN

Initialize_Queues()

Set_Weight(PCIe, 3)

Set_Weight(USB, 1)

Set_Weight(Thunderbolt, 3)

WHILE System_Running = TRUE DO

Update_Request_Queues()

// First handle urgent requests

IF Exists_High_Priority_Request() THEN

Device ← Select_High_Priority_Request()

Grant_Bus(Device)

Service(Device)

Release_Bus(Device)

ELSE

// Weighted fair arbitration

Device ← Weighted_Round_Robin(

PCIe_Queue,

USB_Queue,

Thunderbolt_Queue)

IF Device != NONE THEN

Grant_Bus(Device)

Service(Device)

Release_Bus(Device)

ELSE

Bus_State ← IDLE

END IF

END IF

Update_Starvation_Counters()

Prevent_Starvation()

END WHILE

END

Weighted Round-Robin

Example:

PCIe        → 3 service opportunities

USB         → 1 service opportunity

Thunderbolt → 3 service opportunities

The weights should be configurable according to system requirements.

Starvation Prevention

FUNCTION Prevent_Starvation()

BEGIN

FOR EACH Device DO

IF Device.Wait_Time > MAX_WAIT_TIME THEN

Device.Priority ← Device.Priority + BOOST

Add_To_Urgent_Queue(Device)

END IF

END FOR

END FUNCTION

This ensures that a lower-bandwidth USB device cannot wait indefinitely while high-speed devices continuously request service.

Bus Allocation

Request

↓

Queue

↓

Priority Check

↓

Starvation Check

↓

Weighted Arbitration

↓

Grant Bus

↓

Transfer

↓

Release Bus

Why this achieves Level 4

The algorithm contains:

- Multiple device queues.

- Priority handling.

- Weighted arbitration.

- Starvation prevention.

- Loops.

- Conditional statements.

- Configurable policies.

It balances performance and fairness rather than giving unlimited priority to high-speed devices.

Q5. Hybrid I/O Communication Mechanism

Problem Understanding

Different devices have different data rates.

Low-speed devices do not justify the overhead of DMA setup, while high-speed devices generate enough data to make CPU-driven polling inefficient.

Therefore, the system uses:

Low-speed devices

Polling

High-speed devices

DMA + interrupts

2. Device Classification

Devices

│

Determine Data Rate

│

┌───────┴────────┐

↓                ↓

Low-Speed        High-Speed

↓                ↓

Polling          DMA + IRQ

3. Pseudocode

ALGORITHM Hybrid_IO_Controller

INPUT:

Device_List

OUTPUT:

Device_Data

BEGIN

Initialize_IO_System()

FOR EACH Device IN Device_List DO

IF Device.Data_Rate <= LOW_SPEED_THRESHOLD THEN

Device.Mode ← POLLING

ELSE

Device.Mode ← DMA_INTERRUPT

Configure_DMA(Device)

Enable_DMA_Interrupt(Device)

END IF

END FOR

WHILE System_Running = TRUE DO

// Handle low-speed devices

FOR EACH Device IN Device_List DO

IF Device.Mode = POLLING THEN

IF Device.Is_Ready() = TRUE THEN

Data ← Device.Read()

Process_Data(Device, Data)

END IF

END IF

END FOR

Perform_Normal_CPU_Tasks()

END WHILE

END

4. DMA Interrupt Handler

ISR DMA_Completion_Handler(Device_ID)

BEGIN

Save_Minimum_Context()

Status ← DMA.Get_Status(Device_ID)

IF Status = TRANSFER_COMPLETE THEN

Buffer ← DMA.Get_Completed_Buffer(Device_ID)

Mark_Buffer_Ready(Device_ID, Buffer)

Queue_For_Processing(Device_ID, Buffer)

Clear_DMA_Interrupt(Device_ID)

ELSE IF Status = TRANSFER_ERROR THEN

Record_DMA_Error(Device_ID)

Restart_DMA(Device_ID)

END IF

Restore_Minimum_Context()

RETURN_FROM_INTERRUPT()

END ISR

5. Polling Mechanism

FUNCTION Poll_Low_Speed_Device(Device)

BEGIN

IF Device.Status_Register = READY THEN

Data ← Read_Device_Register(Device)

Process_Data(Device, Data)

Clear_Device_Status(Device)

ELSE

RETURN

END IF

END FUNCTION

Polling is appropriate when:

- Data arrives infrequently.

- Device status checks are inexpensive.

- Interrupt setup overhead would not provide significant benefit.

6. Dynamic Mode Switching

A stronger implementation can dynamically change the device's I/O method based on measured traffic.

ALGORITHM Dynamic_Hybrid_Mode

INPUT:

Device

BEGIN

Data_Rate ← Measure_Data_Rate(Device)

CPU_Load ← Measure_CPU_Load(Device)

IF Data_Rate < LOW_SPEED_THRESHOLD AND

CPU_Load < CPU_LIMIT THEN

Device.Mode ← POLLING

ELSE

Device.Mode ← DMA_INTERRUPT

Configure_DMA(Device)

Enable_Interrupt(Device)

END IF

END

If a device suddenly begins generating large amounts of data, the system can switch it from polling to DMA.

7. Overall Hybrid Architecture

CPU

│

┌───────────┴───────────┐

│                       │

Low-Speed Devices       High-Speed Devices

│                       │

Polling                    DMA

│                       │

│                 Main Memory

│                       │

│                       ↓

│                DMA Completion

│                       │

│                       ↓

└───────────────→ Interrupt

│

↓

CPU Handler

│

↓

Deferred Processing