SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | G.THARAKNATHREDDY | Reg.No.: | 192472032 |
| Department: | CSE-AI | Date: |  |

ANNEXURE A

COMPARATIVE ANALYSIS

- Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

- Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

- Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

- Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

- Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Code of Conduct:

I G.THARAKNATHREDDY(192472032) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: G.THARAKNATHREDDY

MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

ANSWERS:

Q1. Comparison of L1, L2, and L3 Cache

Introduction

CPU cache is a small, high-speed memory located close to the processor. It stores frequently accessed instructions and data so that the CPU does not need to access slower main memory repeatedly. Modern processors generally use multiple cache levels: L1, L2, and L3.

Comparison

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Closest to CPU core | Close to CPU core | Usually shared among cores |
| Capacity | Smallest | Medium | Largest |
| Access latency | Lowest | Low | Higher than L1/L2 |
| Bandwidth | Very high | High | High |
| Throughput | Very high | High | High |
| Cost per bit | Highest | High | Lower than L1/L2 |
| Typical use | Immediate instructions/data | Frequently reused data | Shared/recently used data |
| Sharing | Usually per core | Per core or cluster | Usually shared |
| Miss penalty | Lowest | Moderate | Higher |
| Main benefit | Minimum latency | Balance of speed and capacity | Reduces DRAM accesses |

Latency

The typical relationship is:

L1 < L2 < L3 < DRAM

L1 is designed for extremely fast access because it is located closest to the processor. L2 provides more capacity at slightly higher latency. L3 is larger and often shared, but its access latency is higher.

Exact latency values depend heavily on the processor architecture, clock frequency, cache design, and workload, so fixed numerical values should not be treated as universal.

Bandwidth and Throughput

L1 generally provides the highest effective bandwidth to an individual CPU core because it is extremely close to the execution units. L2 provides substantial bandwidth with greater capacity, while L3 provides a larger shared data pool.

Cache throughput depends not only on cache size but also on:

- Number of cache banks.

- Number of ports.

- CPU frequency.

- Number of cores.

- Cache-line size.

- Memory access pattern.

- Hit rate.

Impact on Processor Performance

Consider a processor executing a program:

CPU Request

↓

L1 Cache

↓ Cache Hit

Data immediately available

If the data is not in L1:

CPU

↓

L1 Miss

↓

L2

↓

L3

↓

DRAM

Every additional level increases the time required to obtain the data.

Critical Analysis

Increasing cache size does not automatically guarantee better performance. A larger cache can improve the hit rate, but it can also increase complexity, area, and sometimes access latency.

Therefore, processors use a hierarchical design:

L1 → very low latency

L2 → latency/capacity balance

L3 → large shared cache

This hierarchy allows the processor to achieve high performance without requiring the entire memory system to be built from expensive SRAM.

Conclusion

L1, L2, and L3 caches form a hierarchy in which L1 prioritizes minimum latency, L2 balances latency and capacity, and L3 provides larger shared storage while reducing DRAM accesses. Efficient cache hierarchy improves CPU throughput by increasing cache-hit rates and reducing processor stalls.

Q2. SRAM vs DRAM

Introduction

SRAM and DRAM are both volatile semiconductor memories, but their internal structures make them suitable for different levels of the memory hierarchy.

SRAM is primarily used for CPU caches, whereas DRAM is primarily used as main memory.

Comparison

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Storage mechanism | Flip-flop cells | Capacitor-based cells |
| Latency | Very low | Higher |
| Bandwidth | Very high in cache applications | High |
| Throughput | Very high for small working sets | High for large data transfers |
| Density | Low | High |
| Cost per bit | High | Lower |
| Refresh | Not required | Required |
| Power | No refresh power, but larger cell area | Refresh required |
| Typical use | L1/L2/L3 cache | Main memory |
| Capacity | Small | Large |

SRAM in Cache Design

SRAM is used for cache because the processor needs very fast access.

CPU

↓

SRAM Cache

↓

DRAM

The small physical size of SRAM cells is a disadvantage in terms of density, but its speed makes it ideal for L1, L2, and L3 caches.

DRAM in Main Memory

DRAM provides much greater capacity at lower cost per bit.

It is suitable for:

- Operating systems.

- Applications.

- Databases.

- Large datasets.

- AI workloads.

However, DRAM requires periodic refreshing and has greater access latency than SRAM.

Latency

The main distinction is:

SRAM → lower latency

DRAM → higher latency

When a CPU finds data in SRAM cache, it can continue execution much faster than if the data has to be retrieved from DRAM.

Bandwidth and Throughput

DRAM can provide extremely high aggregate bandwidth, particularly with multiple memory channels and modern DDR technologies. SRAM provides very high low-latency bandwidth at the cache level.

Therefore, saying simply that "SRAM always has higher bandwidth than DRAM" is not technically sufficient. SRAM has the latency advantage, while modern DRAM can provide enormous aggregate bandwidth because of its high density and multiple channels.

Critical Analysis

SRAM cannot simply replace DRAM because doing so would make large-memory systems extremely expensive and physically inefficient.

Similarly, using only DRAM for caches would increase processor access latency.

The optimal design is:

CPU

↓

SRAM Cache

↓

DRAM Main Memory

↓

SSD/Storage

Conclusion

SRAM is ideal for high-speed cache memory, while DRAM is ideal for large-capacity main memory. Their complementary characteristics allow computer systems to achieve both low latency and large memory capacity.

Q3. Virtual Memory, Paging, and Cache Memory

Introduction

Cache memory and virtual memory are different mechanisms that improve or manage memory access in different ways.

- Cache memory reduces CPU memory-access latency.

- Virtual memory provides an abstraction between virtual and physical addresses.

- Paging is a mechanism used by virtual-memory systems to manage memory in fixed-size pages.

Comparison

| Feature | Cache Memory | Virtual Memory | Paging |
| --- | --- | --- | --- |
| Primary purpose | Reduce access latency | Provide virtual address space | Manage memory in pages |
| Managed mainly by | Hardware | OS + hardware | OS + hardware |
| Basic unit | Cache line | Virtual page | Page |
| Fast storage | SRAM cache | DRAM | DRAM + storage when necessary |
| Normal latency | Very low | Depends on translation/cache state | Can become very high on page fault |
| Storage access | Normally not required | May use storage as backing store | May require SSD/storage |
| Effect on throughput | Usually improves | Improves flexibility/isolation | Excessive paging can reduce throughput |

Cache Mechanism

When the CPU requests data:

CPU

↓

Cache

↓ Hit

Data

If the data is not present:

CPU

↓

Cache Miss

↓

Lower Cache / DRAM

A cache miss increases access time but does not normally require a disk/SSD operation.

Virtual Memory

Virtual memory gives processes a virtual address space.

Virtual Address

↓

Address Translation

↓

Physical Memory

Modern systems commonly use translation caches such as the TLB (Translation Lookaside Buffer) to reduce the cost of repeated address translations.

Paging

Paging divides memory into fixed-size pages.

If a required page is already in DRAM:

Virtual Page

↓

Physical Page in DRAM

If it is not available and has to be fetched from secondary storage:

Page Fault

↓

SSD/Storage

↓

DRAM

↓

CPU

This can be dramatically slower than a cache access.

Impact on Throughput

Cache memory generally improves throughput by reducing CPU stalls.

Paging can improve overall memory utilization, but excessive page faults can severely reduce system throughput. This condition is often called thrashing when the system spends excessive time moving pages instead of executing useful work.

Critical Analysis

Cache and paging should not be treated as competing technologies.

They operate at different levels:

CPU

↓

Cache

↓

DRAM

↓

Virtual-memory backing storage

A well-designed system needs both.

Conclusion

Cache memory primarily addresses processor latency, while virtual memory and paging primarily address memory management, isolation, and capacity. Cache misses generally have much smaller penalties than page faults involving storage. Therefore, sufficient DRAM and effective caching are essential for high system throughput.

Q4. NAND Flash vs DRAM

Introduction

NAND Flash and DRAM are both important memory technologies, but they serve very different purposes.

DRAM is volatile, high-speed working memory, while NAND Flash is non-volatile storage memory.

Comparison

| Feature | DRAM | NAND Flash |
| --- | --- | --- |
| Volatility | Volatile | Non-volatile |
| Latency | Very low compared with storage | Much higher |
| Bandwidth | High | High sequential bandwidth possible |
| Random access | Excellent | Much slower |
| Throughput | High for memory workloads | Good for storage workloads |
| Persistence | No | Yes |
| Refresh | Required | Not required |
| Typical use | Main memory | SSDs, phones, embedded storage |
| Write behavior | Fast memory writes | Program/erase operations are slower |
| Best role | Active working data | Persistent data |

Latency

DRAM is designed for frequent random accesses by the CPU.

NAND Flash is designed primarily for persistent storage. Its access latency is much higher than DRAM, particularly for small random operations.

Therefore:

DRAM → low-latency working memory

NAND → persistent high-capacity storage

Bandwidth

Modern NAND-based SSDs can provide high sequential read/write throughput, especially when multiple NAND channels operate in parallel.

However, storage bandwidth should not be confused with DRAM latency.

For example, a storage device may achieve very high sequential throughput while still having much greater access latency than DRAM.

Hierarchical Memory Design

A typical hierarchy is:

CPU

↓

L1/L2/L3 Cache

↓

DRAM

↓

NAND Flash / SSD

DRAM

Used for:

- Running applications.

- Active datasets.

- CPU working memory.

- AI model execution.

- Database buffer pools.

NAND Flash

Used for:

- Operating-system storage.

- Applications.

- Files.

- Large datasets.

- AI model storage.

Critical Analysis

Replacing DRAM with NAND Flash would significantly reduce cost per bit and provide persistence, but active programs would experience much higher access latency.

Replacing NAND Flash with DRAM would provide faster access but would lose persistence and become much more expensive for large capacities.

Therefore, they are complementary rather than interchangeable.

Conclusion

DRAM should be positioned close to the processor as high-speed working memory, while NAND Flash should provide persistent high-capacity storage.

The hierarchy balances:

DRAM → performance

NAND → capacity and persistence

Q5. MRAM vs RRAM

Introduction

MRAM and RRAM are emerging/non-volatile memory technologies designed to provide persistent data storage with potentially better performance and energy characteristics than traditional storage technologies in selected applications.

Their actual performance varies significantly by implementation, memory generation, process technology, and device architecture.

Comparison

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Full form | Magnetoresistive RAM | Resistive RAM |
| Storage principle | Magnetic state | Resistance state |
| Non-volatile | Yes | Yes |
| Access time | Fast | Fast to moderate depending on implementation |
| Read performance | High | High |
| Write performance | High | Technology-dependent |
| Throughput | High | Potentially high |
| Standby power | Very low | Very low |
| Endurance | Generally high | Device-dependent |
| Density potential | High | Very high potential |
| Main challenge | Cost/density trade-offs | Variability and endurance |
| Possible applications | Embedded memory, caches, persistent memory | Embedded memory, storage, compute-in-memory |

MRAM

MRAM stores information using magnetic states.

A major advantage is that MRAM is non-volatile, meaning data remains stored when power is removed.

Advantages include:

- Fast read access.

- Good endurance.

- Low standby power.

- Non-volatility.

- Suitable for embedded applications.

MRAM can therefore be useful where both fast access and persistence are required.

RRAM

RRAM stores information by changing the resistance of a memory cell.

Potential advantages include:

- High density.

- Non-volatility.

- Low standby power.

- Potential for highly parallel architectures.

- Potential suitability for compute-in-memory AI applications.

However, practical RRAM implementations can face challenges involving:

- Device variability.

- Resistance-state distributions.

- Write reliability.

- Endurance.

- Manufacturing consistency.

Access Time

Both technologies can offer substantially faster access than conventional NAND storage in appropriate implementations.

However, exact access time should not be stated as a universal fixed value because it depends on the specific device architecture.

Throughput

MRAM can provide high read/write throughput for applications requiring fast persistent memory.

RRAM has significant potential for high parallel throughput, especially in specialized architectures.

For AI applications, RRAM is particularly interesting because its physical properties can potentially support compute-in-memory, reducing data movement between processor and memory.

Energy Efficiency

Both technologies can reduce standby energy because they are non-volatile.

A simplified comparison is:

Non-Volatile Memory

│

┌─────────┴─────────┐

│                   │

MRAM                 RRAM

│                   │

Magnetic storage      Resistance storage

│                   │

Fast + durable       Dense + scalable

│                   │

Embedded systems       AI / storage /

compute-in-memory

Critical Analysis

MRAM and RRAM should not be judged only by access time.

A complete evaluation should consider:

- Read latency.

- Write latency.

- Endurance.

- Energy per operation.

- Density.

- Manufacturing cost.

- Data retention.

- Variability.

- Application requirements.

For applications requiring high endurance and predictable fast access, MRAM can be attractive.

For highly dense architectures and specialized AI/compute-in-memory applications, RRAM can be attractive.

Conclusion

MRAM provides a strong combination of speed, endurance, low standby power, and non-volatility, while RRAM offers strong potential in density, low-power operation, and compute-in-memory architectures. The best choice depends on the workload and implementation rather than one technology being universally superior.